| Wyoming State Plan for The Emergency Alert System EAS Plan Updated 12/28/2021 |  |
| --- | --- |
| For information contact: Laura Grott/(307) 632-7622 |  |

Table of Contents

SIGNATURES OF APPROVAL AND CONCURRENCES	4

PURPOSE 	5

INTRODUCTION	5

AUTHORITY	5

ASSUMPTIONS	5

DEFINITIONS	6

OPERATION OF THE EMERGENCY ALERT SYSTEM	8

National-level EAS 	8

State-level EAS	8

Local-level EAS	9

Weather EAS 	9

PROCEDURES FOR PUBLIC OFFICIALS 	9

PROCEDURES FOR BROADCAST AND CABLE MEDIA	 11

MEDIA EAS ACTIVITIES	12

MEDIA RESPONSIBILITIES	12

TESTS

Required Weekly Test (RWT)	13

Required Monthly Test (RMT)	13

Wyoming EAS Scripts and Formats	16

AUTHORITY TO ACTIVATE THE WYOMING STATE EAS SYSTEM	17

NATIONAL ACTIVATION EAS CHART	18

STATE LEVEL EAS CHART	19

LOCAL LEVEL EAS CHART	20

NATIONAL WEATHER RADIO AREAS (With attachment)	21

WYOMING EAS OPERATIONAL AREAS AND LP-1 STATIONS	22

EAS HEADER CODE ANALYSIS	23

WYOMING ORIGINATOR CODES	24

WYOMING “L-CODE” FORMATS	24

Broadcasters	24

Cable TV	24

Weather Service Offices	25

Civil Authorities	25

FCC Event Codes	26

State of Wyoming County Codes	27

WYOMING OPERATIONAL AREA SECTIONS	28

Natrona/Converse Operational Area	28

Laramie Operational Area	29

Sheridan/Johnson Operational Area	30

Sweetwater Operational Area	31

Albany/Carbon/Platte/Goshen/Niobrara Operational Area	32/33

Campbell/Crook/Weston Operational Area	34

Fremont Operational Area	35

Hot Springs/Washakie Operational Area	36

Park/Big Horn Operational Area	37

Uinta/Lincoln/Sublette Operational Area	38

Teton Operational Area	39

Signatures of Approval and Concurrence:

Approval/Concurrence:

Approved:

Wyoming Office of Homeland Security

Lynn Budd	____________________________________________

National Weather Service

Rob Cox	 __________________________________________________

FCC Enforcement Bureau

James Arden Barnett, Jr.

Rear Admiral, USNR (Ret.)

Chief, Public Safety and Homeland

Security Bureau

Federal Communications Commission

Wyoming Association of Broadcasters

Laura Grott

WYOMING STATE EMERGENCY ALERT SYSTEM (EAS) PLAN

Purpose

The purpose of this plan is to provide procedures for statewide activation of the Wyoming State-Level Emergency Alert System (EAS).

Introduction

These procedures were prepared with the help of the Wyoming Office of Homeland Security, National Weather Service, Emergency Operational Centers, Wyoming Association of Broadcasters, State Cable systems, and state and local officials of the State of Wyoming. This plan provides the necessary means and operating procedures for designated state and federal officials to issue emergency information and warning to the general public of Wyoming. Because of the existing commercial and public broadcast coverage Wyoming viewers and listeners will be able to receive emergency information in a timely and concise manner.

Acceptance of or participation in this plan shall not be deemed to prohibit a licensee of cable operator from exercising their independent discretion and responsibility in any given situation. The Federal Communications Commission Rules and Regulations provide station management to use discretionary exercise regarding the transmission of emergency messages to the general public. Broadcast stations and cable systems originating emergency communications shall be deemed to have conferred rebroadcast authority as specified in FCC Rules Section 11.54.

Detailed procedures, agreed upon by the broadcast and cable industries and the local County Emergency Operational Centers, which will permit designated government officials to issue local emergency messages and instructions, via EAS in threatened or actual emergencies, will be published as Local Operational Area Plans.

Authority

Title 47 U.S.C. 151, 154(1) and (o), 303(r), 524(9) and 606; and 47 C.F.R. Part 11, FCC Rules and Regulations, Emergency Alert System (EAS) as pertains to day-to-day emergency operations

Assumptions

The listening and viewing habits of the public are inherent factors of consideration and are conducive to the positive effectiveness of the Wyoming EAS. In times of emergency or crisis most people by instinct or habit will turn on his or her radio or television receiver. However, continued public education is required to assure the public that the broadcast medium is the established method of distributing emergency information from local, state and national sources.

All participating broadcast stations and cable media will monitor NOAA Weather Radio, if possible, as a secondary or “other” assignment. If unable to monitor an assigned input the SECC will work with the station to find an input that is both receivable and suitable. This is NOT a final draft, but should be considered a living, working document. As conditions within the broadcast community and state change so should the document.

All LP-1 stations should have the capability of monitoring at least three audio inputs; 1) Wyoming Public Radio (where available), 2) NOAA Weather Radio, and 3) the local government activation source.

The State of Wyoming has eleven Operational Areas.

The basic monitoring assignments will be as stated in each of the local operational areas.

Definitions

Emergency: A situation posing an extraordinary threat to the safety of life and property. Examples are, but not limited to natural situations such as: floods, heavy snow, high winds, icing conditions, widespread fires, discharge of hazardous materials, tornadoes, widespread power failures, industrial explosions, and civil disorders,

Designated Government Officials: The person or persons designated by governments signatory to this procedure to request activation of the EAS and to make emergency announcements.

EAS Designations: These are the FCC's EAS Station Designations reflecting the EAS status of every broadcaster and cable operator.

National Primary (NP) - Source of all national EAS Alerts. This station will be monitored by designated LP stations.

State Primary (SP) - A primary source of EAS State programming which can originate with the governor or designated representative, such as a state's emergency operations officer. (WOHS)

State Relay (SR) – Will primarily be provided by National Weather Service. Future state relay options would possibly include the following: satellite service, Wyoming Public Radio and Wyoming Public Television. They will relay those state EAS Messages that are received from the Wyoming Office of Homeland Security; when and if they become involved. They can also relay national, local, and weather alerts to those areas where no other means is available.

Local Primary (LP-1) - The local broadcast station in a defined operational area that is designated the primary station to transmit EAS messages. LP-1 stations are primarily sources of Local Area Emergency EAS Messages. They can also relay national, state, and weather alerts.

Local Primary (LP-2) - The local broadcast station in a defined operational area that is designated as the backup to the LP-1 Station in some operational areas. LP-2 stations are secondary sources of Local Area Emergency EAS Messages. They can also relay national, state, and weather alerts.

Participating National (PN) - Most normal broadcasters and cable operators are designated as "PN". These sources are for delivering all levels of EAS to the public.

Non-Participating National (NN) - Broadcasters who hold a NN Authorization from the FCC to sign off the air during a National Emergency.

Emergency Operation Center (EOC) -  (State, County, City. etc.) A specifically designed and designated facility within a jurisdiction used for Emergency Operations.

National Warning System (NAWAS) - A dedicated Homeland Defense telephone system, accessible from specific Local Area Control Point (LACP) and Weather Service officials, the Wyoming Highway Patrol and Wyoming Office of Homeland Security in the State of Wyoming.

National Oceanographic and Atmospheric Administration (NOAA) - NOAA Weather Radio stations are encoding their alerts with the coding that is used for EAS. Broadcasters and cable operators can feed their EAS Decoders with the audio from any normal NOAA Weather Radio receiver, and their EAS Decoder will react just as it does with broadcaster EAS codes.

Local Area Control Point (LACP)  - At least one EOC Center with 24-hour answering point, with NAWAS in each area should ultimately have an EAS Encoder to send local alerts to the Area LP-1 station, and all other broadcasters and cable operators that want to receive it directly. If an EAS encoder/decoder cannot be procured, a manual call to the LP-1 will be used until such time as it can be implemented at a future date.

WYOMING STATE EAS OPERATIONAL PLAN

Operation of the Emergency Alert System

National Level EAS

In a national emergency, the White House directs activation of the Emergency Alert System (EAS) to provide the President with a means of addressing the public on very short notice. The activation is accomplished by disseminating an Emergency Action Notification (EAN) message via a dedicated network relaying information to control points of the national Primary Entry Point (PEP) network.  In addition, radio and TV broadcast networks, telephone common carriers are voluntary participants. Non-participating national stations (NN) so designated by the FCC must go off the air.  Participating National Stations (PN) and cable systems will continue as outlined in the appropriate checklist and Part 11 of the FCC Rules and Regulations until receipt of an Emergency Action Termination (EAT) message, at which time all stations will resume normal broadcasting. Stations originating emergency communications shall be deemed to have conferred rebroadcast authority (see Appendix C).

State-Level EAS

The Wyoming EAS is activated by a request from authorized officials to the State’s EAS State Primary (Wyoming Office of Homeland Security - WOHS). Local Primary sources in the local EAS area should monitor dissemination of state levels emergency information to all other stations, cable systems and the public.

Activation of the State-Level EAS will be from the following:

Governor

Officer-on-duty, Wyoming Office of Homeland Security

NWS offices with county warning responsibility in Wyoming (pg. 17)

Activation of the State-Level Amber Alert will be by those officials listed as follows:

Wyoming Highway Patrol

Local-Level EAS

Activation of the Local EAS will be by the National Weather Service or officials as designated in the Local Operational Area Plan. Normally, the request would be made by the PSAP (Public Safety Answering Point or NAWAS Warning Point), or directly to the Local Primary (LP1) source serving the affected area(s).

Upon receipt of an (Local) Area EAS request for activation, all broadcast stations and cable systems, which are voluntarily participating, may conduct, at the discretion of management, operations in accordance with the provisions of the State EAS or operational area plan. EAS may be activated for day-to-day emergencies posing a threat to life or property. For unique emergency situations involving local areas, authorities may request EAS activation through the Local Activation of the State-Level EAS will be from the following:

Governor

Director, Wyoming Office of Homeland Security

EOC Center in the Local Area

NWS offices with county warning responsibility in Wyoming (pg. 17)

Primary (LP) source serving the affected EAS local area.

Weather EAS Procedures

Each NWS office with county warning responsibility in Wyoming will provide EAS activation for specific weather alert messages. These messages are mainly of a WATCH, WARNING or ADVISORY nature (e.g. tornado, severe thunderstorm, flash flood, winter storm, etc.). In addition, NWS offices can provide EAS activation for authorized messages from designated officials listed in this plan.

Dissemination of authorized messages will be directly through NWS NOAA Weather Radio broadcasts and EMWIN (Emergency Manager Weather Information Network).

For unique emergency situations involving local areas, authorities at the county emergency operation center (EOC) may request EAS activation through the LP1 and LP2 stations serving the affected EAS local area.

Procedures for Public Officials

Upon receipt of a state or operational level activation request, all broadcast stations and cable systems may conduct, at the discretion of management, operations in accordance with the provisions of the Wyoming State EAS Plan. Day-to-day emergencies posing a threat to the safety of life and property which would cause activation of the state level EAS include, but are not limited to, floods, heavy snow, high winds, icing conditions, widespread fires, discharge of toxic gases, tornadoes, widespread power failures, industrial explosions, Child Abduction Emergencies and civil disorders. In some instances, a state level EAS activation will be issued through the NWS, from the State Primary, which is the state emergency operations center in Cheyenne, Wyoming (WOHS). WOHS will then relay the alert by telephone to the all county EOCs by phone/NAWAS for distribution to LP-1s.

Interim Activation: When the State WYSOC (WOHS) issues an EAS message from the state emergency operations center, the message will be relayed the to National Weather Service. Designated officials will use the following format when contacting the key station(s):

“This is _______________________ of ___________________________________.  I request that the Emergency Alert System be activated in the State of Wyoming”  (Followed by a description of emergency situation).

When the authorized person requests activation of the Wyoming State EAS System, they should provide the following information.

_Who the requesting official is.

_A call back request, using a number previously provided to the control point or other established authentication procedures.

_Broadcast details (i.e., live, recorded; immediate or delayed).  Program material should be provided covering the following points:

_What areas are involved, or the entire state.

_Situation summary  (describe the nature of the emergency).

_Action taken by local governments.

_Instructions or messages to the public.

(An authorized public official may also activate the EAS through the NWS alerting system.)

Procedures for Broadcast and Cable Media

Upon receipt of a request to activate the EAS at the state level or operational level by civil authority, the operator at the State Primary Source will authenticate and begin recording, if necessary, all emergency messages and proceed as follows:

1.	Broadcast the following announcement:

“We interrupt this program to provide the following emergency instructions."

2.	Transmit the Emergency Alert System header codes and Attention Signal (see Appendix H).

3.	Broadcast the following announcement and broadcast the emergency material:

“We interrupt this program to activate the Emergency Alert System in the State of Wyoming at the request of_________________.  The emergency situation is___________________and affects (entire state, portion of state).”

Repeat nature of emergency to allow time for other broadcasters to respond.

4.	Broadcast the emergency information or common program as received from the activating official. Include the source of information and the time received.

5.	Repeat as necessary.

6.	When received, broadcast Emergency Action Termination by making the following announcement:

"This concludes emergency programming under the Wyoming State Emergency Alert System.  All broadcast stations and cable systems may now resume normal operations."

*Actions 3 through 6 should be less than 2 minutes from receipt of message.

7.	 Send the End of Message (EOM) code (FCC Regulations, Sec. 11.31).

8.	Log or record all activity of the EAS system per FCC Regulations (Sec.11.55).

Each broadcast station and cable system, upon receipt of a State Level or operational level emergency action notification will perform, at the discretion of management, the same procedures outlined above. The emergency information may be automatically or manually rebroadcast (FCC Rules, Sec II.51k) from the monitored source or the information may be presented by station personnel.

All broadcast stations and cable systems must be able to receive a Common Alerting Protocol (CAP) source as required by the FCC Rules and Regulations 47 C.F.R. Part 11.56  www.fema.gov/common-alerting-protocol.

Television stations shall transmit a visual message containing the Originator, Event, Location, and valid time period of an EAS message.  If the message is a video crawl, it shall be displayed at the TOP of the television screen or where it will not interfere with other visual messages. (Requirements for cable systems are outlined in FCC rules, Sec. 11.51g.)

Broadcasters and cable systems have the option of transmitting only the EAS header and EOM codes without the attention signal and emergency announcement. This is acceptable to quickly relay EAS coded messages through areas unaffected by the emergency.

Media EAS Activities

Monitor the primary station designated as the LP-1 (see Appendix B to Appendix I) for your Operational Area for receipt of any further instructions. When possible, monitor the NOAA Weather Radio frequency serving your area. Where NWR reception is not possible, monitor the state relay network. LP-l stations should monitor NWR and a State Primary or State Relay frequency and a local public safety frequency, when it is assigned.

Media Responsibilities

The State Primary, State Relay and LP-1 stations should monitor the NP/SP/NOAA All-Hazards Radio, Common Alerting Protocol data from the FEMA/IPAWS aggregator to provide the required national EAS link to the system.

A procedure should be developed by the stations for discontinuing normal operations and broadcasting the alert message, live or recorded either automatically or manually as permitted in the rules (FCC rules, Sec. 11.51)

After receiving notification of the termination of the State-level EAS, participating media may resume regular operations in accordance with their authorization (see EAS Checklist). During emergencies, operation outside normal parameters is allowed. Such operations should be logged.

Each station should ensure that required EAS equipment is properly installed and programmed with appropriate event codes.

Tests

Assure that EAS equipment is operating properly.

1.	All stations are required to record information relevant to all EAS tests (FCC rules, Sec 11.61); AM stations should refer to 73.1250.

2.	All stations (except Class D FM and Low Power television) must transmit, once a week at random days and times, a Required Weekly Test (RWT) consisting of EAS Header Codes and EOM codes.

3.	All stations must transmit a required monthly test (RMT) consisting of EAS Header Codes, Attention Signal, Test Script, and EOM code within 60 minutes of receipt of a test from a Local or State Primary source (Class D FM and Low Power TV transmit only the test script).

Tests of State Emergency Alert facilities, as coordinated between the Wyoming SECC Chair and state officials, shall be on a random or scheduled basis from a point, which would originate the common emergency program.

4.	The following requirements regarding both RWTs and RMTs apply to all cable operators and all broadcasters, “PN”, as well as “NN” stations. Even stations that have elected not to participate in local EAS alerts, must still rebroadcast their local RMT every month. There are two exceptions to these rules.  First, Class “D” FM and LPTV stations need not have an EAS Encoder. They must have an EAS Decoder. Thus, these stations are exempt from running the weekly digital code RWT test. However, they must retransmit monthly RMT tests as outlined below, minus the EAS Header Codes and Attention Signal. In addition, LPTV stations must present all EAS information visually, just as all other TV stations must do. The second exception is for FM Translator and TV Translator stations, which are not required to have any EAS equipment.

Required Weekly Test (RWT)

1.	Transmission: All broadcasters and cable operators must transmit an RWT once each week at random days and times except for the week of the RMT test. There are no time-of-day restrictions. This is a 10.5-second test, consisting only of the EAS Header and End-of-Message Codes.

2.	Reception: All broadcasters and cable operators receiving a RWT from one of their monitored sources must log receipt of this test from their one or two required monitored sources. No further action is required.

Required Monthly Test (RMT)

1.	Transmission: RMTs are to be initiated by the SR, and LP-1 and LP-2 stations. During some months, the test will actually be initiated by the 911-Center or Emergency Operation Center associated with these stations in this State Plan. During the designated week for this test, all other broadcasters and cable operators are to wait for this test and then react as described in (4.) below. These tests shall always use the Event Code “RMT”, never codes such as “State Test”, or “Local Area Test”, etc.

2.	Scheduling of RMTs / Week and Time-of-Day:

·	RMTs shall always occur during the first, full, Sunday-thru-Saturday week of the month.

·	Time frame and origination of the RMT tests shall adhere to the following format:

| MONTH | TIME FRAME | STN. | ORIGINATING SOURCE |
| --- | --- | --- | --- |
| JANUARY | DAY / 8:30 AM to Local Sunset | LP | LP-1 |
| FEBRUARY | NIGHT / Local Sunset to 8:30 AM | LP | LP-1 |
| MARCH | DAY / 8:30 AM to Local Sunset | LP | LP-1 |
| APRIL | NIGHT / Local Sunset to 8:30 AM | LP | LP-1 |
| MAY | DAY / 8:30 AM to Local Sunset | LP | LP-1 |
| JUNE | NIGHT / Local Sunset to 8:30 AM | LP | LP-1 |
| JULY | DAY / 8:30 AM to Local Sunset | LP | LP-1 |
| AUGUST | NIGHT / Local Sunset to 8:30 AM | LP | LP-1 |
| SEPTEMBER | DAY / 8:30 AM to Local Sunset | LP | LP-1 |
| OCTOBER | NIGHT / Local Sunset to 8:30 AM | LP | LP-1 |
| NOVEMBER | DAY / 8:30 AM to Local Sunset | LP | LP-1 |
| DECEMBER | NIGHT / Local Sunset to 8:30 AM | LP | LP-1 |

NOTES:

LP = Test will come from the Local Primary station that you monitor.

Wyoming STATE EOC = The State EOC will send the RMT at its discretion.

The LP and PN stations must then rebroadcast this test within 60 minutes of receiving it.

3.	Scheduling of RMTs/Recommended Time Constraints:

LP stations, and the State EOC are requested to use judgment in the scheduling of times for RMTs.  Since all broadcasters and cable operators are required to rebroadcast this test within 60 minutes of receiving it, care should be taken to not put undue hardship on TV broadcasters in particular, when they are carrying their highest-revenue programming. On a daily basis, these periods would include all major newscasts: early morning, noontime, evening, and late evening.  In addition, the times of major events are recommended to be avoided, such as: pre-planned Presidential speeches, hours of a major national or local news story carried outside of normal newscast hours, local and national election coverage, and

major sporting events like World Series games and the Super Bowl.

Broadcasters and cable operators who have a complaint regarding the scheduling of RMTs in their area should make their concerns known to their area Chair (see “The Wyoming SECC” section in this Plan for names). If a satisfactory resolution is not reached at that level, the State EAS Chairs should be contacted.

4.	Reception/ re-transmission of RMTs

All broadcasters and cable operators receiving an RMT test must re-transmit this test within 60 minutes of receiving the test.  [For Daytime-only stations receiving nighttime RMT, this test must be re-transmitted within 60 minutes of the Daytime-only station’s sign-on.] Transmission of this RMT test takes the place of the Required Weekly Test (RWT). Times should be logged for both the receipt and re-transmission of the RMT test. Broadcast and cable management should impress upon their staff that re-transmission of this test is not an option. It is a FCC violation to fail to re-transmit this test within 60 minutes of receiving it. The best policy may be to set your EAS unit for a 60-minute automatic countdown upon receiving an RMT. If the operator on duty does not send the test manually within that window, the box will do it for him when time runs out. If it is not possible to retransmit the RMT upon sign on, transmit an RWT in place of the RMT. As there are several LPs in the state monitoring stations outside of their Operational Area there is an exception to this rule. A RMT received from outside your Operational Area does not need to be forwarded. It should be properly logged and a comment that it is outside of the stations Operational Area should be made in the log.

5.	Time-Duration and County-Location Codes to be used

TIME-DURATION codes used in the EAS Header Code for all EAS Tests shall be 30 MINUTES.

COUNTY-LOCATION codes used in the EAS Header Code for EAS Tests shall conform to these guidelines:

6.	Miscellaneous

SP & SR Stations: All tests, RWT and RMT, shall use the Location Code for the entire state.

LP Stations: All tests, RWT and RMT, shall include the Location Code for all counties in that LP station’s Local Area of responsibility.  To determine the counties in their Local Area of responsibility, each LP station should consult the “Boundary Map of Wyoming EAS Local Areas”, and/or the cover sheet for the “FCC Mapbook”, both found in the Appendix of this Plan. Under the new EAS Plan, some counties have been moved compared to the old EBS Plan. Please read carefully.

PN and NN Stations and Cable Operators: RMT tests shall be re-transmitted unchanged, except for the “L-Code”. Thus, RMTs will include all counties present in the original message. For the RWT originated each week by each PN and NN station, and each cable operator, the county-location code used shall be the county for the broadcaster’s City of License, or cable operator’s Community of License.  Other counties in the station’s/system’s service area may be added at management discretion.

Wyoming  EAS Test Scripts and Formats

The following test scripts and formats shall be used by all Wyoming broadcasters, cable operators, and emergency agencies when originating EAS tests.

1.	RWT: No script is used for the RWT. Entire test takes 10.5 seconds.  Format is as follows:

·	Stop regular programming

·	one-second pause

·	Send EAS Header Code 3 times

·	one-second pause

·	Send EAS End-of-Message Code 3 times

·	one-second pause

·	Resume normal programming

22222.	RMT: LP stations and emergency agencies originating this test should use the following format. All other broadcasters and cable operators will receive the test in this format, and must re-transmit it within 60 minutes in the same format. Format is as follows:

·	Stop regular programming

·	Optional Intro: “This is a test of the (Local Area)  Wyoming Emergency Alert System.”

·	one-second pause

·	Send EAS Header Code 3 times (All sources must use Event Code “RMT” for this test.)

·	one-second pause

·	Send EAS Attention Signal (8 seconds)

·	Read Test Script: “This is a test of the (Local Area) Wyoming Emergency Alert System. In the event of an emergency, this system would bring you important information. This test is now concluded.”

·	one-second pause

·	Send EAS End-of-Message Code 3 times

·	one-second pause

·	Resume normal programming

Timing Note: The script above can be read in 9-10 seconds. All other elements of the RMT (the Header Codes and an 8-second Attention Signal) take from 19-21 seconds to complete (that length depending on the number of county codes contained in the Header). The goal of writing this short script was to fit the entire test into a 30-second time period. LP stations and emergency agencies should make every attempt to complete this test within 30 seconds. Pre-recording the script at the length needed to achieve this goal would probably be helpful.

Script Note: (Local Area)  = LPs: Use the name of your Local Area found in Appendix I. SPs use the phrase, “State of Wyoming”.

LIST OF THOSE WITH THE AUTHORITY TO ACTIVATE

THE WYOMING STATE EAS SYSTEM

GOVERNOR

Mark Gordon	(307) 777-7434

DIRECTOR, WYOMING OFFICE OF HOMELAND SECURITY

Lynn Budd	(307) 777-4900

### METEOROLOGIST-IN-CHARGE, NATIONAL WEATHER SERVICE

Rob Cox, Cheyenne	 (307) 772-2468 ext. 642

Chris Jones, Riverton 	(307) 857-3898 ext. 642

David Billingsley, Salt Lake City, UT	(801) 524-4378 ext. 222
David Hintz, Rapid City, SD	(605) 341-9271

Keith Meier, Billings, MT	(406) 652-0851

WARNING COORDINATION METEOROLIGISTS, NATIONAL WEATHER SERVICE

Jared Allen, Cheyenne 	(307) 772-2468 ext. 726
 Tim Troutman, Riverton	(307)  857-3898 ext. 726

Kevin Barjenbruch, Salt Lake City, UT	(801) 524-5113
 Susan K. Sanders, Rapid City, SD 	(605) 341-9271
 Tom Frieders, Billings, MT	(406) 652-0851

National Weather Service

Wyoming Signal Distribution

###### See attached map

Cheyenne	162.550 MHz  	Afton	162.475 MHz

# Casper	162.400 MHz	Cody	162.400 MHz

Lander	162.475 MHz	Worland.	162.500 MHz

Sheridan	162.475 MHz	Thermopolis.	162.500 MHz

Rawlins	162.425 MHz	Glendo Resv.	162.450 MHz

Rock Springs	162.550 MHz	Terry Peak, SD	162.525 MHz

N. of Evanston	162.450 MHz	Dubois	162.450 MHz

Grant Village	162.450 MHz	Kaycee	162.550 MHz

Jackson	162.525 MHz	Mammoth	162.425 MHz

Kemmerer	162.525 MHz	Gillette	162.500 MHz

Pinedale	162.500 MHz	Elk Plaza, Yellowstone NP…162.425 MHZ

Newcastle	162.475 MHz

Wyoming State EAS Operational Areas

EAS Header Code Analysis

An EAS Header Code contains the following elements, sent in the following sequence:

·	[Preamble] ZCZC-ORG-EEE-PSSCCC+TTTT-JJJHHMM-LLLLLLLL-

·	Attention Signal

·	Aural, Visual, or Text Message

·	[Preamble] NNNN

[Preamble] = (Clears the system) - Sent automatically by the Encoder.

ZCZC = (Start of ASCII Code) - Sent automatically by the Encoder.

ORG = (Originator Code) - Preset once by user, then sent automatically by the Encoder.  See following Section (B.) for code you must use.

EEE = (Event Code) - Determined by user, each time an alert is sent.  See following Appendix H for the  codes to be used in Wyoming.

PSSCCC = (County-Location Code) - Determined by user, each time an alert is sent.  See following Appendix H for the assigned codes of all Wyoming counties.

TTTT = (Duration of Alert) - Determined by user, each time an alert is sent.

JJJHHMM = (Date/Time-of-Day) - Sent automatically by the Encoder.

LLLLLLLL = (8-Character ID, Identifying the Broadcaster, Cable TV, Weather Service Office,  Nuclear/Industrial Plant, or Civil Authority operating that Encoder.)   Preset once by user, then sent automatically by the Encoder.  See following below for the format to be followed by all users in constructing their “L-Code”.

Attention Signal - Must be sent if aural, visual, or text message is sent.

[Preamble] = (re-clears the system) - Sent automatically by the Encoder when the End-of-Message sequence is initiated.

NNNN = (End-of-Message Code) - Must be initiated manually at the end of every EAS Alert originated by all sources.  A failure of the system will occur if this code is not sent to reset the Decoders of all stations/operators that carried that alert.

Wyoming Originator Codes

Wyoming Originator Codes Dakota Originator Codes

Following are the Originator Codes for State activation of the EAS.

WXR - To be used by National Weather Service Offices.

CIV - To be used by Emergency Government, Sheriffs, and all other Civil Authorities.

EAS - To be used by all Broadcasters and Cable TV Operators.

Whether used under the authority of the State EAS Plan, or any of the County/Local Area EAS Plans, the following are the Event Codes to be used in the State of Wyoming.  No codes can be added without FCC approval.  County/Local Area EAS Plans that desire to use a code not on this list should submit that code request to the Wyoming Association of Broadcasters for SECC and FCC approval and subsequent addition to this list.  This list will be maintained as a “Master List” for all Event Codes used in the State of Wyoming.

Wyoming  “L-Code” Formats

This 8-character code is affixed to every EAS message originated or re-transmitted by every EAS Encoder. The code identifies the particular broadcaster, cable operator, Weather Service Office, nuclear/industrial plant, or civil authority operating that Encoder. “L-Code” ID’s must adhere to the following formats. No deviation from these formats is allowed, since using certain other characters would cause an error in the system.

###### Broadcasters:

Single Station: “KXXX(FM)

Two Stations:  “KXXXKYYY”

Three or more Stations: The call letters of one of the stations or the abbreviated name of the parent company  is sufficient. All other stations sending the alert should keep a log of alerts sent, as should the ID station. (Per FCC)

###### Cable TV:

(To be determined by the to-be-named SECC Cable TV Chair.)

Weather Service Offices

Use the NWS office’s location identifier, followed by a “/” and the letters NWS.

Examples:    	Cheyenne		KCYS/NWS

Riverton		KRIW/NWS

Rapid City		KUNRNWS

Billings		KBYZ/NWS

Salt Lake City           KSLC/NWS

Civil Authorities: This code uses three components in constructing its 8-character code: Portion of “L-Code”       Source of Characters

State: 	Office of the Governor = WYO GOV

Wyoming Office of Homeland Security = WOHS

###### Local Government:

First four characters = First four letters of name of jurisdiction (Name of County, City, etc.)

Next two characters = Abbreviation for type of jurisdiction:

For County use “CO”

For City use “CY”

For Town use “TN”

For Village use “VL”

For Township use “TP”

For Municipality use “MY”

Last two Characters = Abbreviation for type of agency:

For Sheriff use “SH”

For Fire Dept. use “FD”

For Police Dept. use “PD”

For Traffic Authority use “TA”

For Emergency Services use “ES”

For Emergency Management use “EM”

Examples: Laramie County Sheriff = “LARACOSH”, City of Casper Police = “CASPCYPDWyoming County-Location Codes (“PSSCCC”)

The first digit (“P”) can be used to indicate one-ninth of the county code it precedes, using the following pattern

There are no FCC-Approved Special Wyoming Event Codes at this time.

ORIGINATOR AND EVENT CODE TRANSLATIONS

Originator Codes:

| Org. Code | English Translation |
| --- | --- |
| EAS | Broadcast Station or Cable System |
| EAN | National Emergency Action Notification Network |
| CIV | Civil Authority |
| WXR | The National Weather Service |
| PEP | Primary Entry Point |

| Event Code | English Translation | Model 943 Event Code No. | Event Code | English Translation | Model 943 Event Code No. |
| --- | --- | --- | --- | --- | --- |
| EAN | National Emergency Action Notification | N/A | HWA | High Wind Watch | 21 |
| EAT | National Emergency Action Termination | N/A | HWW | High Wind Warning | 22 |
| NIC | National Information Center Message | N/A | HUA | Hurricane Watch | 23 |
| NPT | National Periodic Test | N/A | HUW | Hurricane Warning | 24 |
| RMT | Required Monthly Test | 33 | HLS | Hurricane Statement | 25 |
| RWT | Required Weekly Test | 34 | LEW | Law Enforcement Warning | 26 |
| ADR | Administrative Message | 1 | LAE | Local Area Emergency | 27 |
| AVA | Avalanche Watch | 2 | NMN | Network Message Notification | 28 |
| AVW | Avalanche Warning | 3 | TOE | 911 Telephone Outage Emergency | 29 |
| BZW | Blizzard Warning | 4 | NUW | Nuclear Power Plant Warning | 30 |
| CAE | Child Abduction Emergency | 5 | DMO | Practice/Demo Warning | 31 |
| CDW | Civil Danger Warning | 6 | RHW | Radiological Hazard Warning | 32 |
| CEM | Civil Emergency Message | 7 | SVA | Severe Thunderstorm Watch | 35 |
| CFA | Coastal Flood Watch | 8 | SVR | Severe Thunderstorm Warning | 36 |
| CFW | Coastal Flood Warning | 9 | SVS | Severe Weather Statement | 37 |
| DSW | Dust Storm Warning | 10 | SPW | Shelter in Place Warning | 38 |
| EQW | Earthquake Warning | 11 | SMW | Special Marine Warning | 39 |
| EVI | Evacuation Immediate Notice | 12 | SPS | Special Weather Statement | 40 |
| FRW | Fire Warning | 13 | TOA | Tornado Watch | 41 |
| FFW | Flash Flood Warning | 15 | TOR | Tornado Warning | 42 |
| FFS | Flash Flood Statement | 16 | TRA | Tropical Storm Watch | 43 |
| FLA | Flood Watch | 17 | TRW | Tropical Storm Warning | 44 |
| FLW | Flood Warning | 18 | TSA | Tsunami Watch | 45 |
| FLS | Flood Statement | 19 | TSW | Tsunami Warning | 46 |
| HMW | Hazardous Materials Warning | 20 | VOW | Volcano Warning | 47 |
| FFA | Flash Flood Watch |  | WSA | Winter Storm Watch | 48 |
|  |  |  | WSW | Winter Storm Warning | 49 |

Wyoming County-Location Codes ("PSSCCC")

The first digit ("P") can be used to indicate one-ninth of the county code it precedes, using the following pattern:

0 = Entire County

| 1 = NW | 2 = NC | 3 = NE |
| --- | --- | --- |
| 4 = WC | 5 = C | 6 = EC |
| 7 = SW | 8 = SC | 9 = SE |

Examples: for all of Natrona County, the Location code would be "056025". Most County-Location Codes in the state of Wyoming will be constructed to indicate the entire county.

STATE OF WYOMING COUNTY CODES

State of Wyoming (used to activate the entire state)	56000

| Albany County | 056001 |
| --- | --- |
| Big Horn County | 056003 |
| Campbell County | 56005 |
| Northern Campbell County NWS | 256005 |
| Central Campbell County NWS | 556005 |
| Southern Campbell County NWS | 856005 |
| Carbon County | 056007 |
| Converse County | 056009 |
| Crook County | 056011 |
| Fremont County | 056013 |
| Goshen County | 056015 |
| Hot Springs County | 056017 |
| Johnson County | 056019 |
| Laramie County | 056021 |
| Laramie County excluding City of Cheyenne | 156021 |
| Laramie County including City of Cheyenne | 256021 |
| Lincoln County | 056023 |
| Natrona County | 056025 |
| Niobrara County | 056027 |
| Park County | 056029 |
| Platte County | 056031 |
| Sheridan County | 056033 |
| Sublette County | 056035 |
| Sweetwater County | 056037 |
| Teton County | 056039 |
| Uinta County | 056041 |
| Washakie County | 056043 |
| Weston County | 056045 |
| Yellowstone National Park (Uses Park County) | 056029 |

Natrona/Converse Local Area

### Local Area 1

(This Area includes the towns and cities within Natrona and Converse Counties.  Casper, Evansville, Mills, Bar Nunn, Midwest, Edgerton, Alcova, Douglas, Glenrock,)

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| Charter | CASPER | Cable | PN | LP1 | NOAA | WY PR |
| KASS-FM | CASPER | 106.9 MHZ | PN | LP1 | NOAA | WY PR |
| KCSP-FM | CASPER | 90.3 MHZ | PN | LP1 | NOAA | WY PR |
| KCWY-TV | CASPER | CH. 13 | PN | LP1 | NOAA | WY PR |
| KDAD FM | BAR NUNN | 92.5 MHZ | PN | LP1 | NOAA | WY PR |
| KFCB – LP | DOUGLAS | 105.1 | PN | KKTY FM | NOAA | WY PR |
| KFNB-TV | CASPER | CH. 20 | PN | LP1 | NOAA | WY PR |
| KGWC-TV | CASPER | CH. 14 | PN | LP1 | NOAA | WY PR |
| KHOC-FM | CASPER | 102.5 MHZ | PN | LP1 | NOAA | WY PR |
| KKRR FM | CASPER | 88.3 MHZ | PN | KTWO TV | KUWR |  |
| KKTL AM | CASPER | 1400 KHZ | PN | LP1 | NOAA | WY PR |
| KKTS AM | Evansville | 1580 KHZ | PN | KWYY | NOAA |  |
| KKTS FM KKTS AM | DOUGLAS EVANSVILLE | 99.3 MHZ 1580 MHZ | PN PN | KWYY KWYY | NOAA NOAA |  |
| KKTY-AM | DOUGLAS | 1470 KHZ | PN | KWYY | NOAA |  |
| KLWC FM | CASPER | 89.1 | PN | KWYY | KUWC/WPR | NWS |
| KMLD FM | CASPER | 94.5 MHZ | PN | LP1 | NOAA | WY PR |
| KPTW TV | CASPER | Ch. 8 | PN | LP1 | NOAA | WY PR |
| KQLT-FM | CASPER | 103.7 MHZ | PN | LP1 | NOAA | WY PR |
| KRNK FM | CASPER | 96.7 MHZ | PN | LP1 | NOAA | WY PR |
| KRVK FM | VISTA WEST | 107.9 MHZ | PN | LP1 | NOAA | WY PR |
| KTED FM | CASPER | 100.5 MHZ | PN | LP1 | NOAA | WY PR |
| KTRS-FM | CASPER | 104.7 MHZ | PN | LP1 | NOAA | WY PR |
| KTWO-AM | CASPER | 1030 KHZ | LP1 | NOAA | WY PR | LOAPC & Natrona EOC |
| KTWO-TV | CASPER | Ch. 2 (17) | LP2 | LP1 | NOAA | WY PR |
| KUYO-AM | EVANSVILLE | 830 KHZ | PN | LP1 | NOAA | WY PR |
| KVOC-AM | CASPER | 1230 KHZ | PN | LP1 | NOAA | WY PR |
| KWYY-FM | MIDWEST | 95.5 MHZ | PN | LP1 | NOAA | WY PR |
| KZQL FM | CASPER | 105.5 MHZ | PN | LP1 | NOAA | WY PR |
| Vyve Broadband | DOUGLAS | Cable | PN | KWYY | EOC BY PHONE | WY PR |

*WY PR – Wyoming Public Radio

Natrona/Converse Local Area

Local Area 1

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Natrona | Sheriff’s Office | (307) 235-9300 |
| Converse | Sheriff’s Office | (307) 358-4700 |

NOTE:  Please contact the Sheriff’s Office in order to contact the county Emergency Management Coordinator

Laramie Local Area

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| Charter | CHEYENNE | Cable | PN | LP1 | LP2 | NOAA |
| KAZY FM | CHEYENNE | 93.7 MHZ | PN | LP1 | LP2 |  |
| KFBC-AM | CHEYENNE | 1240 KHZ | LP1 | NOAA | LP2 | KRRR FM |
| KGAB-AM | CHEYENNE | 650 KHZ | PN | LP1 | NOAA |  |
| KGSC LPTV | CHEYENNE | CH. 47 (1) | PN | LP1 | NOAA |  |
| KGSC-LD | CHEYENNE | 668.3 MHZ | PN | LP1 | NOAA |  |
| KGWN-TV | CHEYENNE | CH. 5 | PN | LP1 | LP2 | NOAA |
| KIGN FM | BURNS | 101.9 MHZ | PN | LP1 | NOAA |  |
| KJJL-AM | CHEYENNE | 1370 KHZ | PN | LP1 | LP2 | NOAA |
| KKAW FM | ALBIN | 107.3 MHZ | PN | LP1 | WY PR | NOAA |
| KKPL FM | CHEYENNE | 99.9 MHZ | PN | LP2 | NOAA | KUNC/CO 91.5 |
| KKTU-TV | CHEYENNE | CH. 33 | PN | LP1 | NOAA | KTWO AM |
| KKWY AM | CHEYENNE | 1630 KHZ | PN | LP1 | LP2 | NOAA |
| KLEN-FM | CHEYENNE | 106.3 MHZ | PN | LP1 | NOAA |  |
| KLWV FM | LARAMIE (CHUGWATER) | 90.9 MHZ | PN | KUWR | KCGY FM | NWS |
| KLWY-TV | CHEYENNE | CH. 27 | PN | KTWO AM | NOAA | WY PR |
| KOLT FM | WARREN AFB | 92.9 MHZ | PN | LP1 | LP2 | WY PR |
| KOLZ-FM | CHEYENNE | 100.7 MHZ | LP2 | LP1 | NOAA |  |
| KRAE-AM | CHEYENNE | 1480 KHZ | PN | LP1 | LP2 |  |
| KRAN FM | WARREN AFB | 103.3MHZ | PN | LP1 | LP2 |  |
| KRRR FM | CHEYENNE | 104.9 MHZ | PN | LP1 | - LP2 |  |
| KXBG FM | CHEYENNE | 97.9 MHZ | PN | LP2 | NOAA | KUNC |
| KYOY FM | HILLSDALE | 92.3 MHZ | PN | LP1 | LP2 | NOAA |
| Pine Bluffs Community TV System | PINE BLUFFS | Cable | PN | KGWN TV Channel 5 | LP2 |  |

Local Area 2 (This area includes the towns and cities within Laramie County, Cheyenne and Pine Bluffs.

*WYPR – Wyoming Public Radio

Laramie Local Area

Local Area 2

NOTE:  Please contact the Sheriff’s Office in order to contact the county Emergency Management Coordinator

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Laramie | Sheriff’s Office | (307) 633-4711 |

Sheridan/Johnson County Local Area

Local Area 3

(This Area includes the towns and cities within Sheridan and Johnson Counties.  Sheridan, Ranchester, Dayton, Story, Buffalo, Kaycee, and Barnum.)

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| Charter | BUFFALO | Cable | PN | KBBS | KLGT FM | NOAA |
| Charter | SHERIDAN | Cable | PN | KROE AM | KZWY FM |  |
| KBBS-AM | BUFFALO | 1450 KHZ | LP1B | NOAA AND EOC BY PHONE | WY PR |  |
| KHRW FM | SHERIDAN | 92.7 MHZ | PN | LP1 | NOAA |  |
| KLGT-FM | BUFFALO | 96.5 MHZ | LP1 | NOAA AND EOC BY PHONE | WY PR |  |
| KLQQ FM | CLEARMONT | 104.9 MHZ | PN | LP1 | NOAA | WY PR |
| KROE-AM | SHERIDAN | 930 KHZ | PN | LP1 | NOAA |  |
| KSGW-TV | SHERIDAN | CH. 12 | PN | LP1 | NOAA |  |
| KSWY TV | SHERIDAN | Ch. 29 | PN | LP1 | NOAA |  |
| KWCF FM | SHERIDAN | 89.3 MHZ | PN | KYTI FM | WY PR |  |
| KWYO-AM | SHERIDAN | 1410 KHZ | PN | LP1 | NOAA |  |
| KYTI FM | SHERIDAN | 93.7 MHZ | PN | LP1 | NOAA |  |
| KZWY-FM | SHERIDAN | 94.9 MHZ | PN | LP1 | NOAA |  |
| KZZS FM | STORY | 98.3 MHZ | LP1A | NOAA AND EOC BY PHONE | WY PR |  |
| Tongue River Cable TV | Ranchester, Dayton, Story | Cable | PN | LP1 | WY PR |  |

##### *WYPR = Wyoming Public Radio

##### Sheridan/Johnson County Local Area

Local Area 3

NOTE:  Please contact the Sheriff’s Office in order to contact the county Emergency Management Coordinator

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Sheridan | Sheriff’s Office | (307) 672-3453 |
| Johnson | Sheriff’s Office | (307) 684-5581 |

Sweetwater Local Area  Local Area 4

(This Area includes the towns and cities within Sweetwater County.  Rock Springs, Green River, Wamsutter and Reliance.)

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| KZWB FM | GREEN RIVER | 97.9 MHZ | PN | LP1A KQSW | NOAA | WY PR |
| KFRZ-FM | GREEN RIVER | 92.1 MHZ | PN | LP1A KQSW | NOAA | WY PR |
| KUGR-AM | GREEN RIVER | 1490 KHZ | PN | LP1A KQSW | NOAA | WY PR |
| KUWX FM | PINEDALE | 90.9 MHZ | PN | NOAA/EAS | LP1 |  |
| KPIN-FM | PINEDALE | 101.1 MHZ | PN | WY PR | NOAA |  |
| KRKK-AM | ROCK SPRINGS | 1360 KHZ | LP1 | NOAA/EAS | LP2 |  |
| KREO FM | JAMES TOWN | 93.5 MHZ | PN |  |  |  |
| KMRZ FM | ROCK SPRINGS | 106.7 MHZ | PN | LP1 | NOAA |  |
| KYCS-FM | ROCK SPRINGS | 95.1 MHZ | LP2 | LP1A KQSW | NOAA | WY PR |
| KGWR-TV | ROCK SPRINGS | CH 13 | PN | KTWO AM | NOAA | WY PR |
| KQSW-FM | ROCK SPRINGS | 96.5 MHZ | LP1A | LP1 | LP2 | WY PR |
| KSIT-FM | ROCK SPRINGS | 99.7 MHZ | PN | LP1 | LP2 | WY PR |
| KUWZ-FM | ROCK SPRINGS | 90.5 MHZ | PN | LP1 | LP2 | WY PR |
| Sweetwater Cable | ROCK SPRINGS GREEN RIVER | Cable | PN | LP1 | LP2 | WY PR |
| *KRAN FM | WAMSUTTER | 97.5 MHZ | PN | LP2 | NOAA | WY PR |
| KFZE FM | DANIEL | 104.3 MHZ | PN | WY PR | NOAA |  |
| KWCN FM | PINEDALE | 89.9 MHZ | PN | WY PR | NOAA |  |

*WYPR = Wyoming Public Radio

Sweetwater Local Area

### Local Area 4

NOTE:  Please contact the Sheriff’s Office in order to contact the county Emergency Management Coordinator

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Sweetwater | ### Sheriff’s Office | (307) 872-6350 |
| Sublette | Sheriff’s Office | (307) 367-4378 |

Albany/Carbon/Platte/Goshen Niobrara County Local Area

Local Area 5

(This Area includes the towns and cities within Albany and Carbon Counties.  Laramie, Centennial, Woods Landing, Rock River, Rawlins, Sinclair, Hanna, Medicine Bow, Saratoga, Encampment, Shirley Basin, Chugwater, Torrington, Lingle, Fort Laramie, Guernsey, Lusk and Lamont.)

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| Charter | LARAMIE | Cable` | PN | LP1 | LP2 | NOAA |
| Charter | RAWLINS | Cable | PN | KTGA FM | NOAA | KBDY FM |
| KANT FM | GUERNSEY | 104.1 MHZ | PN | NOAA | WY PR |  |
| KARS FM | LARAMIE | 102.9 MHZ | PN | LP1 | NOAA | KUNC/CO 91.5 |
| KBDY FM | HANNA | 102.1 MHZ | PN | LP1 | NOAA |  |
| KBDY FM | SARATOGA | 102.1 FM | PN | LP1 | NOAA |  |
| KCGY-FM | LARAMIE | 95.1 MHZ | LP2 | LP1 | NOAA | WY PR |
| KERM FM | TORRINGTON | 98.3 MHZ | PN | KUWR PR | NOAA |  |
| KFNR-TV | RAWLINS | CH 11 (9) | PN | KTWO AM | NOAA | WY PR |
| KGOS AM | TORRINGTON | 1490 KHZ | PN | KUWR PR | NOAA |  |
| KHAT AM | LARAMIE | 1210 KHZ | PN | LP1 | NOAA | WY PR |
| KIMX-FM | LARAMIE | 96.7 MHZ | PN | LP1 | NOAA | WY PR |
| KIQZ-FM | RAWLINS | 92.7 MHZ | PN | LP1 | NOAA | WY PR |
| KKTY-FM | GLENDO | 100.1 MHZ | PN | KWYY | NOAA |  |
| KLMI FM | LARAMIE | 106.1 MHZ | PN | LP1 | KOWB | NOAA |
| KLWV FM | LARAMIE | 90.9 MHZ | PN | LP1 | NOAA |  |
| KOWB-AM | LARAMIE | 1290 KHZ | PN | LP1 | NOAA | WYPR |
| KPAD FM | WHEATLAND | 107.5 MHZ | PN | LP1 | LP2 |  |
| KRAL-AM | RAWLINS | 1240 KHZ | PN | LP1 | NOAA | WY PR |
| KRQU FM | CHUGWATER | 99.5 MHZ | PN | LP1 | LP2 |  |
| KTGA FM | SARATOGA | 99.3 MHZ | PN | LP1 | NOAA |  |
| KUSZ FM | LARAMIE | 93.9 MHZ | PN | LP1 | NOAA | WY PR |
| KUWR-FM | LARAMIE | 91.9 MHZ | LP1 | NPR Squawk Channel | NOAA | EOC |
| KWYP-DT | LARAMIE | CH 8 | PN | LP1 | NOAA |  |
| KYCN-AM | WHEATLAND | 1340 KHZ | PN | NOAA/EOC | KUWR |  |
| KZEW FM | WHEATLAND | 101.7 MHZ | PN | NOAA/EOC | WY PR |  |
| Pine Bluffs Community TV System | FT. LARAMIE | Cable | PN | KGWN TV Channel 5 | KSTF Channel 10 | NOAA |
| Pine Bluffs Community TV System | GUERNSEY | Cable | PN | KGWN TV Channel 5 | KSTF Channel 10 | NOAA |

##### *WYPR = Wyoming Public Radio

##### Albany/Carbon County Local Area NOTE:  Please contact the Sheriff’s Office in order to contact the county Emergency Management Coordinator

### Local Area 5

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Albany | Sheriff’s Office | (307) 721-5322 After Hrs. Dispatch (307) 721-2526 |
| Platte | Sheriff’s Office | (307) 322-2331 |
| Goshen | Sheriff’s Office - (307) 532-4026 | After Hrs. Torrington PD Dispatch (307) 532-7001 |
| Niobrara | Sheriff’s Office | (307) 334-2240 |
| Carbon | Sheriff’s Office | (307) 324-2776 |

Campbell/Crook/Weston County Local Area

Local Area 6

(This Area includes the towns and cities within Campbell, Crook and Weston Counties. Gillette, Wright, Moorcroft, Sundance, Upton and Newcastle.)

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| Charter | GILLETTE | Cable | PN | LP1A | LP1 | WY Public Radio |
| Charter | NEWCASTLE | Cable | PN | KASL | KGWY FM | WY Public Radio |
| KAML-FM | GILLETTE | 97.3 MHZ | LP1A | NOAA | EOC | WY Public Radio |
| KASL-AM | NEWCASTLE | 1240 KHZ | PN | LP1B | NOAA Newcastle | WY Public Radio |
| KDDV FM | WRIGHT | 101.5 MHZ | LP1C | NOAA | EOC | WY Public Radio |
| KGWY-FM | GILLETTE | 100.7 MHZ | LP1B | NOAA | EOC | WY Public Radio |
| KHAD FM | UPTON | 104.5 MHZ | PN | LP1 | NOAA | WY Public Radio |
| KIML-AM | GILLETTE | 1270 KHZ | LP1 | NOAA | EOC/UHF | WY Public Radio |
| KLED FM | Antelope Valley | 93.3 MHZ | LP1D | NOAA | EOC | WY Public Radio |
| KLOF FM | GILLETTE | 88.9 MHZ | PN | LP1B | KUWG 90.9 NPR | NWS |
| KLSX FM | ROZETTE | 99.1 MHZ | PN | LP1 | NOAA | WY Public Radio |
| KLWD FM | GILLETTE | 91.9 MHZ | PN | LP1B | NOAA | Wyoming Public Radio |
| KQOL FM | SLEEPY HOLLOW | 105.3 MHZ | PN | LP1 | LP1B | NWS |
| KXXL FM | MOORCROFT | 106.1 MHZ | PN | LP1 | NOAA | WY Public Radio |
| KYDT-FM | SUNDANCE | 103.1 MHZ | PN | NOAA, Lead, SD | KHSD TV, Lead SD |  |
| Tongue River Cable TV | Hulett, Pine Haven, Osage, Sundance, Midwest | Cable | PN | LP1 |  | WY Public Radio |

*WYPR = Wyoming Public Radio

Campbell/Crook/Weston County Local Area

Local Area 6

NOTE:  Please contact the Sheriff’s Office in order to contact the county Emergency Coordinator

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Campbell | Sheriff’s Office | (307) 682-7271 |
| Crook | Sheriff’s Office | (307) 283-1225 |
| Weston | Sheriff’s Office | (307) 746-4444 |

Fremont County Local Area

Local Area 7

(This Area includes the towns and cities within Fremont County. Lander and Riverton.)

*After digital transition Feb 2009

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| Charter | RIVERTON/ LANDER | Cable | PN | LP1 | NOAA |  |
| KCWC-DT | RIVERTON | CH 8 | PN | LP1 | LP2 | NOAA |
| KCWC-TV | LANDER | CH 4 | PN | LP1 | LP2 | NOAA |
| KDLY-FM | LANDER | 97.5 MHZ | LP1 | NOAA | EOC BY PHONE | WY PR |
| KDNO FM | RIVERTON | 101.7 MHZ | PN | LP1 | LP2 | WY PR |
| KFNE-TV | RIVERTON | CH 10 | PN | KTWO AM | NOAA | WY PR |
| KGWL-TV | LANDER | CH 5 (7)* | PN | KTWO AM | NOAA | WY PR |
| KOVE-AM | LANDER | 1330 KHZ | PN | LP1 | LP2 | NOAA |
| KRKM FM | FT. WASHAKIE/LANDER | 91.7 | PN | LP2 | KUWW/WPR | NOAA |
| KTAK-FM | RIVERTON | 93.9 MHZ | LP2 | LP1 | NOAA |  |
| KFCW FM | RIVERTON | 93.1 MHZ | PN | LP1 | LP2 | NOAA |
| KTUG FM | HUDSON | 105.1 MHZ | PN | LP1 | LP2 | NOAA |
| KVOW-AM | RIVERTON | 1450 KHZ | PN | LP1 | LP2 | NOAA |
| KWYW FM | RIVERTON | 99.1 MHZ | PN | LP1 | LP2 | NOAA |

Fremont County Local Area  *WYPR – Wyoming Public Radio

NOTE:  Please contact the Sheriff’s Office in order to contact the county Emergency Coordinator

Local Area 7

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Fremont | Sheriff’s Office | (307) 332-5611 |

Hot Springs/Washakie County Local Area

Local Area 8

(This Area includes the towns and cities within Hot Springs and Washakie Counties. Thermopolis, Shoshoni, Worland.)

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| Charter | THERMOPOLIS | Cable | PN | LP1 | NOAA |  |
| KUDA FM | THERMOPOLIS | 88.7 MGH | PN | EOC BY PHONE | NOAA | WY PR |
| KWOR-AM | WORLAND | 1340 KHZ | LP1A | KZMQ FM | NOAA | WY PR - EOC |
| KKLX-FM | WORLAND | 96.1 MHZ | LP1 | KZMQ FM | NOAA | WY PR – EOC |
| KYTS FM | MANDERSON | 105.7 MHZ | LP1B | KZMQ FM | NOAA | WY PR - EOC |
| Charter | WORLAND | Cable | PN | KKLX FM | NOAA | EOC/PHONE |

*WYPR = Wyoming Public Radio

HOT SPRINGS/WASHAKIE COUNTY LOCAL AREA
Area 8

NOTE: Please contact the Sheriff’s Office in order to contact the county Coordinator

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Hot Springs | Sheriff’s Office | (307) 864-2622 |
| Washakie | Sheriff’s Office | (307) 347-2242 |

##### Park/Big Horn County Local Area

Local Area 9

(This Area includes the towns and cities within Park and Big Horn Counties. Cody, Powell, Lovell, Greybull, Basin and Byron.)

| Station | CODY | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| Charter | CODY | Cable | PN | LP1 | NOAA |  |
| Charter | GREYBULL | Cable | PN | LP1 | NOAA |  |
| Charter | LOVELL, BASIN, BYRON, GREYBULL, COWLEY, OTTO, SHELL & MANDERSON | Cable | PN | LP1 | NOAA |  |
| KBEN FM | COWLEY | 103.3 | LP2 | LP1 | WY PR |  |
| KCGL FM | POWELL | 104.1 MHZ | LP1D | LP2 | NOAA | WY PR |
| KCWB FM | BYRON | 92.1MHZ | LP1D | LP2 | NOAA | WY PR |
| KFGR FM | POWELL | 88.1 MHZ | PN | LP1 | WY PR |  |
| KNWT FM | CODY | 89.1 MHZ | PN | LP1 | WY PR |  |
| KODI-AM | CODY | 1400 KHZ | LP1A | LP2 | NOAA | WY PR |
| KOFG FM | CODY | 91.1 MHZ | PN | LP1 | WY PR |  |
| KPOW-AM | POWELL | 1260 KHZ | PN | LP1 | NOAA |  |
| KROW FM | CODY | 101.1 MHZ | PN | LP1 | KUWP |  |
| KTAG-FM | CODY | 97.9 MHZ | LP1B | LP2 | NOAA | WY PR |
| KUWP | POWELL | 90.1 MHZ | PN | NOAA | WY PR & EOC | KEMC – MT PR |
| KWHO FM | LOVELL | 107.1 MHZ | PN | LP1 | WY PR |  |
| KZMQ-AM | GREYBULL | 1140 KHZ | LP1C | LP2 | NOAA | WY PR |
| KZMQ-FM | GREYBULL | 100.3 MHZ | LP1 | LP2 | NOAA | WY PR |
| Lovell Cable | LOVELL | Cable | PN | LP1 | KUWP |  |
| TCT West | LOVELL BASIN, BYRON, GREYBULL, BASIN, COWLEY, OTTO, SHELL, MANDERSON | Cable | PN | LP1 | EOC BY PHONE |  |

*MT PR – Montana Public radio    *WY PR – Wyoming Public Radio

PARK/BIG HORN COUNTY LOCAL AREA

Local Area 9

NOTE: Please contact the Sheriff’s Office in order to contact the county coordinator

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| KRSV-AM | AFTON | 1210 KHZ | PN | KMTN FM | WY PR | NOAA |
| KRSV-FM | AFTON | 98.7 MHZ | PN | KMTN FM | WY PR | NOAA |
| KDWY FM | DIAMONDVILLE | 105.3 MHZ | PN | LP1 | KQSW FM |  |
| KEVA-AM | EVANSTON | 1240 KHZ | DARK | DARK | DARK |  |
| KUWE FM | EVANSTON | 89.7 MHZ | LP1 | *See Area 5 |  |  |
| KBMG FM | EVANSTON | 106.1 MHZ | PN | WY PR | NOAA |  |
| KADQ FM | EVANSTON | 98.3 MHZ | PN | KDWY | WY PR |  |
| KNYN FM | EVANSTON | 99.1 MHZ | PN | KDWY | WY PR |  |
| All West Comm. | EVANSTON | Cable | PN | LP1 | KQSW FM |  |
| KMER-AM | KEMMERER | 950 KHZ | PN | LP1 | KQSW FM |  |
| KAOX FM | KEMMERER | 107.3 MHZ | PN | LP1 | KQSW FM |  |
| KTYN FM | THAYNE | 91.9 MHZ | PN | WY PR | NOAA |  |

Lincoln/Uinta Local Area

Local Area 10

(This Area includes the towns and cities within Lincoln and Uinta Counties. Evanston, Kemmerer, Afton, Mountain View, and Cokeville.)

Lincoln/Uinta Local Area - *WYPR = Wyoming Public Radio

Local Area 10

NOTE:  Please contact the Sheriff’s Office in order to contact the county Coordinator

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Lincoln | Sheriff’s Office | (307) 877-3971 |
| Uinta | Sheriff’s Office | (307) 783-1000 |

Teton Local Area

Local Area 11

(This Area includes the towns and cities within Teton County. Jackson, Wilson, and Moran Junction.)

| Station | Location | Frequency | EAS Designation | Primary EAS Assignment | Secondary EAS Assignment | Other Assignment |
| --- | --- | --- | --- | --- | --- | --- |
| Charter | JACKSON | Cable | PN | LP1 | NOAA |  |
| KHOL FM | JACKSON | 89.1 KHZ | PN | LP1 | NOAA | WY PR |
| KJAX FM | JACKSON | 93.5 MHZ | PN | LP1 | NOAA | WY PR |
| KMLT FM | JACKSON | 88.3 MHZ | PN | LP1 | KUWJ 90.3 | NOAA |
| KMTN-FM | JACKSON | 96.9 MHZ | PN | LP1 | NOAA | WY PR |
| KSGT-AM | JACKSON | 1340 KHZ | PN | LP1 | NOAA | WY PR |
| KUWJ-FM | JACKSON | 90.3 MHZ | PN | LP1 | NOAA | WY PR |
| KZJH-FM | JACKSON | 95.3 MHZ | LP1 | NOAA/EOC BY PHONE | WY PR |  |

TETON LOCAL AREA - *WYPR = Wyoming Public Radio
Area 11

| County | Authority to Activate | Phone Number |
| --- | --- | --- |
| Teton Teton Teton | Emergency Management EOC Sheriff’s Dispatch | 307-413-5040 307-733-9572 307-733-2331 |
| Yellowstone National Park Dispatch | Dispatch | 307-344-2640 |

Wyoming State EAS Plan

Multilingual EAS Alerting SUMMARY

Appendix A

Added:  April 30, 2018

In compliance with Section 11.21 € of the Code of Federal Regulations, the Wyoming SECC compiled information submitted in 2017 by EAS participants throughout the state of Wyoming with regard to their current or planned efforts to provide emergency alert content in languages other than English to non-English speaking audiences.  These submissions were made in response to Section 11.21(d) of the Code of Federal Regulations and provided the SECC with information regarding the availability of multilingual EAS alerts to the public across the state of Wyoming.

The following is submitted in order for the FCC “to ensure that the Commission has sufficient and accurate information on any existing and planned activities by EAS Participants to provide EAS alert content in languages other than English to their non-English speaking audiences.”

No participants reported carrying foreign-language alerts, nor do they have any plans to do so.

According to the US Census, 92.8% of Wyoming’s residents speak only English at home.  As a result, the National Weather Service, Wyoming Highway Patrol (for Amber Alerts) do not issue alerts in languages other than English.